SignaSense

Technologies Used in the Project

Smart Glove, Smart Walking Stick and Mobile Application

| Project name | SignaSense Assistive Communication and Navigation System |
| --- | --- |
| Document purpose | Separate summary of the hardware, software, communication, recognition and development technologies used in the project. |
| Prepared for | Final year project documentation and presentation support |
| Date | May 2026 |

This document can be attached as a supporting technology appendix or used as a standalone technology summary.

# 1. Introduction

SignaSense is an assistive technology project made up of three main parts: a smart glove for sign-language-based communication, a smart walking stick for obstacle detection and navigation guidance, and an Android mobile application that supports visual captions, speech output, glove connection, camera sign backup and user-defined signs.

The technologies used in the project were selected to support low-cost implementation, offline operation, simple maintenance and practical demonstration. The design combines embedded systems, mobile software, Bluetooth communication, sensor data processing, computer vision and local user interface technologies.

# 2. Technology Stack Summary

| Area | Technology Used | Main Role in SignaSense |
| --- | --- | --- |
| Embedded controller | ESP32 development board | Controls the smart glove and smart walking stick, reads sensors, runs firmware logic and handles Bluetooth/Wi-Fi features. |
| Smart glove sensing | Five flex sensors, resistor voltage dividers, ESP32 ADC pins | Reads finger bending patterns and converts them into finger states, letters, words and suggestions. |
| Smart stick sensing | Ultrasonic distance sensor and buzzer | Measures obstacle distance and provides backup sound alerts when Bluetooth audio is unavailable. |
| Bluetooth audio | ESP32 Bluetooth Classic A2DP source and Bluetooth speaker | Sends embedded voice guidance from the smart stick to speakers such as SINOBAND Book and EWA Audio A20. |
| Mobile communication | Bluetooth Low Energy and local Wi-Fi page prototype | Transfers glove readings to the phone and supports local testing without mobile data. |
| Mobile application | Android SDK, Kotlin, Java 17, AndroidX, Material Components | Provides onboarding, glove connection, captions, spoken output, camera backup, theme selection and user-defined sign training. |
| Camera recognition | CameraX and MediaPipe Tasks Vision | Uses the phone camera as a backup sign detection mode through hand tracking and visual gesture recognition. |
| Local intelligence | Rule-based recognition, calibration, smoothing, suggestion logic and local storage | Improves noisy sensor readings, builds words from detected letters and keeps user-defined sign data on the device. |

# 3. Hardware Technologies

The project uses affordable and available hardware components so that the prototype can be demonstrated, repaired and improved without relying on expensive specialist equipment.

| Hardware Technology | Use in the Project |
| --- | --- |
| ESP32 board | Main microcontroller for both the smart glove and smart stick. It provides analog inputs, digital I/O, Bluetooth, Wi-Fi and enough processing capability for the prototype firmware. |
| Flex sensors | Used on the glove to sense finger bending. Each sensor changes resistance when bent, allowing the ESP32 to estimate whether a finger is straight, half-bent or bent. |
| Voltage divider resistors | Used with each flex sensor to produce a measurable analog voltage for the ESP32 ADC. This is necessary because a flex sensor alone does not produce a useful voltage signal. |
| Ultrasonic sensor | Used on the smart stick to measure the distance between the user and an obstacle. The firmware converts echo pulse duration into centimetres and metres. |
| Buzzer | Used as a backup alert device on the smart stick when the Bluetooth speaker is not connected. |
| Bluetooth speaker | Used to output spoken guidance from the smart stick. The current firmware supports EWA Audio A20 and SINOBAND Book speaker names. |
| Android phone | Runs the SignaSense application, displays captions, provides camera backup recognition and produces speech for the glove interface. |

# 4. Embedded Software Technologies

The embedded firmware is written for Arduino IDE compatibility. This approach makes the code easier to upload, test and modify on common ESP32 development boards.

| Embedded Technology | Purpose |
| --- | --- |
| Arduino C/C++ | Used to write the firmware for the smart glove and smart stick. |
| ESP32 Arduino core | Provides ESP32-specific functions such as analogRead, GPIO control, Bluetooth, Wi-Fi and non-volatile storage. |
| ADC reading and filtering | Used in the glove to read flex sensor values, smooth noise and classify bends. |
| Ultrasonic pulse timing | Used in the stick to calculate distance from echo time and apply calibration constants. |
| ESP32 Preferences/NVS | Used on the stick to store boot count and alternate between English and Luganda voice modes across power cycles. |
| ESP32-A2DP library | Used to make the ESP32 act as a Bluetooth audio source and stream embedded voice clips to a speaker. |
| Embedded audio clips | Used instead of cloud text-to-speech. English and Luganda clips are stored in firmware and played as ordered guidance messages. |
| BLE GATT service | Used by the glove to send JSON-like readings and recognized letter information to the Android application. |

# 5. Mobile Application Technologies

The Android app is the main user interface for deaf users and also supports accessibility features such as spoken output, visual captions and dark/light theme selection. It is designed to work locally with the glove and camera backup functions.

| Mobile Technology | Purpose |
| --- | --- |
| Android SDK | Base platform for building and installing the SignaSense mobile application. |
| Kotlin | Main programming language for the application activities and logic. |
| Java 17 | Compilation target used by the Android project. |
| Android Gradle Plugin 8.5.2 and Gradle | Build system used to compile the app and generate APK files. |
| AndroidX Core, AppCompat and Activity KTX | Support libraries used for modern Android app structure and compatibility. |
| Material Components | Used for cleaner buttons, controls, layout styling and theme support. |
| Android WebKit | Supports embedded web-style views and local device pages where required. |
| Android TextToSpeech | Used by the app to speak captions, committed words and selected messages. |
| Bluetooth and BLE APIs | Used to scan, connect and communicate with the smart glove. |
| SharedPreferences/internal local storage | Used for app settings, theme selection, calibration and user-defined signs that remain on the current phone. |

# 6. Camera and Recognition Technologies

The camera backup is included so that the system still has a communication option when the glove battery is low or when a demonstration requires visual sign recognition. It is a backup mode, while the glove remains the main sensing device for the project.

| Recognition Technology | Use |
| --- | --- |
| CameraX | Provides camera preview, camera lifecycle handling and frame access inside the Android app. |
| MediaPipe Tasks Vision | Provides hand landmark and vision task support for camera-based sign detection. |
| Model files in the models folder | Used as research/training resources for hand gesture recognition and future improvement of the camera mode. |
| Rule-based letter mapping | Used to convert glove bend states into letters when the sensor pattern is stable. |
| User-defined sign training | Allows a user to train only the letters they need on their own phone. These mappings stay local and do not affect other users. |
| Word suggestion logic | Uses detected letters to propose words and sentences before speech output, reducing accidental communication from noisy letters. |
| Motion detection concept | Supports gesture motions such as greeting or direction-related signs in the camera backup path. |

# 7. Communication Technologies

Different communication methods were used because the smart stick and smart glove have different needs. The stick focuses on direct audio output to a speaker, while the glove focuses on sending sensor data to the phone application.

| Communication Technology | Role |
| --- | --- |
| Bluetooth Classic A2DP | Used by the smart stick to connect directly to a Bluetooth speaker and output voice guidance without a phone app. |
| Bluetooth Low Energy | Used by the smart glove to send low-power sensor readings to the Android app. |
| Wi-Fi access point/local web page | Used during prototype testing for local display and debugging without mobile data. |
| USB serial monitor | Used for firmware upload, live debugging, calibration and checking raw sensor readings. |
| Offline local operation | The core glove/stick functions do not require internet data. The system relies on local Bluetooth, local phone processing and embedded firmware logic. |

# 8. Development and Testing Tools

| Tool | Use in Project Development |
| --- | --- |
| Arduino IDE / Arduino CLI | Used to compile and upload ESP32 firmware. |
| Android Studio | Used to develop, build and install the Android application. |
| ADB | Used for phone installation, testing and screenshot capture. |
| Serial Monitor | Used to view distance values, raw flex sensor values, Bluetooth status and calibration output. |
| PowerShell | Used for running local build, test and file management commands on the development computer. |
| Python tooling | Used for document generation, diagram preparation, audio processing and supporting project automation. |
| Microsoft Word and PowerPoint | Used for the report, diagrams and presentation materials. |

# 9. Version Snapshot from the Current Project

| Item | Version / Setting |
| --- | --- |
| Android compile SDK | 36 |
| Android minimum SDK | 26 |
| Android target SDK | 34 |
| Kotlin plugin | 1.9.24 |
| Android Gradle Plugin | 8.5.2 |
| Java target | 17 |
| CameraX libraries | 1.4.1 |
| MediaPipe Tasks Vision | 0.10.14 |
| ESP32 Arduino core installed | 1.0.6 |
| ESP32-A2DP library installed | 1.4.0 |

# 10. Key Design Notes and Conclusion

The project is designed for mostly offline operation. The smart stick uses embedded English and Luganda voice clips instead of cloud text-to-speech, and the buzzer remains a backup when the Bluetooth speaker is not connected. The glove depends on voltage divider wiring for reliable flex sensor readings, while the app keeps user-defined signs local to the current phone so that custom signs do not affect other users. The camera sign mode is included as a backup/support feature, and the one-sensor stick guidance requires the user to scan left and right so the firmware can compare available paths.

In summary, SignaSense combines embedded electronics, Android mobile development, Bluetooth communication, sensor processing and local recognition logic. These technologies support obstacle guidance, sign detection, captions and speech output while keeping the prototype affordable, demonstrable and practical for final-year presentation.